Guidelines for auditing Grid CAs version 1.0-b5

Status of This Memo

This memo provides information to the Grid community on the guidelines for auditing Grid CAs.  It does not define any standards or technical recommendations. Distribution is unlimited.

Copyright Notice

Copyright © Openl Grid Forum (2007).  All Rights Reserved.

Abstract

Grids use X.509 certificates for authentication and authorization.  These certificates are typically issued by Certificate Authorities operated by real institutions that compose a virtual organization.  In order to ensure compliance with established policies and operational procedures, and to recommend necessary changes in controls, policies or procedures, these Certificate Authorities should be externally audited.  This document provides the audit checklist which describes auditing items and acceptable evidence for the verification of these items.  As guidelines for auditing Grid CAs, detailed processes of auditing are also described in this document.

- Introduction

Grids use X.509 certificates for authentication and authorization.  Those certificates are typically issued by Certificate Authorities (CAs) operated by real institutions that compose a virtual organization.  In order to ensure compliance with established policies and operational procedures, and to recommend necessary changes in controls, policies or procedures, these Certificate Authorities should be externally audited.  Processes of auditing include an independent examination of documentation, records, activities to access the adequacy of system controls, interviewing of the staffs in charge of administration and operation of the Certificate Authority, and inspection of evidence and physical devices, etc.  The audit checklist is built based on Audit checklist for Grid CAs Version 4.1.  This document provides the audit checklist which describes auditing items and evidences for the verification.  As guidelines for auditing Grid CAs, detailed processes of auditing are also described in this document.

It should be noted that it was determined that the OGF mission would not include responsibility (and thereby liability) to either vet certificate policies (CPs) or audit CAs.  The purpose of this paper is to show guidelines for auditing Grid CAs and does not mean that the OGF would accept responsibility and liability.

- Procedures of auditing

  - A list of marks for auditing

Auditors prepare a list of marks for auditing, which is a table of the audit checklist, evidence, procedures and results of the examination, and scores of the items in the audit checklist.  Each item in the audit checklist should be scored according to the results of the examination.  For example, each item can be scored from A to D, and X as below.

- A:  Good.

- B:  Recommendation (minor change)

- C:  Recommendation (major change)

- D:  Advice (must change)

- X:  Could not evaluate (N/A)

  - Materials used for auditing

Auditing consists of a pre-examination, a main examination, and a post examination.  In the pre-examination, all possible documents available for the auditors are examined.  The followings are examples of such documents.

- CP/CPS

- Manuals for subscribers (e.g. enrollment manual)

- Operational manuals (for CA operators)

- CA Repository (e.g. Web site)

- CA Certificate

- CRL

- End entity certificates

- HSM manual (or appropriate web site)

- Any other document described as“published on the repository”in the CP/CPS

- Any other document available for the auditors

Some of these documents must be available on the repository and the auditors can request that the CA provides the other documents.

In the main examination, the auditors visit the CA and interview the staff in charge of the administration and operation of the CA, and inspect evidence and physical devices, etc.  The followings are examples of items that may be inspected.

- CA room

- CA machine including HSM

- A backup media of the CA private key and its place (e.g. a safe box).

- Offline media (e.g. a sealed envelope) which contains a pass phrase of the CA private key and its place (e.g. a safe box).

- Media storage of archived logs and other documents and their place (e.g. a safe box).

- End entity certificates (if not available for the pre-examination)

- Logs of the CA/RA servers

- Logs of the CA repository (e.g. Web server)

- Records of operation of the CA private key (including accesses to the HSM)

- Access log to the CA room

- Any other documents (e.g. daily report of the CA operators)

  - Pre-Examination

In the pre-examination, the auditors evaluate each item in the audit checklist by examining appropriate materials available for the auditors for the pre-examination.  Some of the items could be scored according to the results of the pre-examination, but some of the items may need an interview to the CA operators and inspections to be scored.  Necessary evidences for the evaluation depend on available materials and their usefulness for the pre-examination.  If the auditors are unable to score an item, the auditors should describe necessary examinations, interviews, and inspections in the list of marks for auditing, which will be carried out during the main examination.

  - Main Examination

In the main examination, the auditors visit the CA, interview the CA staff, and inspect documents (e.g. archived logs) and equipment (e.g. CA server, HSM, backup media, etc) according to the results of the pre-examination.  The auditors should score the items which could not be scored in the pre-examination.

  - Post-Examination

In the post-examination, the auditors draft an auditing report according to the results of the pre-examination and the main examination.  The audit report should include the followings:

- Date of auditing

- Terms of subjects of auditing

- Names of the auditors

- Names of the participants

- Results of the auditing

  - Scores of the items in the audit checklist

  - Comments for Scores B, C, and D.

The auditing report should be drafted and sent to the CA within few days after the auditing.  The CA is expected to send a report on the plans for improving the CA operation to the auditors within a few weeks.

- Auditing Checklist

This section shows an auditing checklist.  For each item, evidence and methods for the evaluation such as the section number of the CP/CPS, subjects of the inspections and issues to be interviewed, are described.  The indicated section numbers of the CP/CPS are according to RFC2527.  Sections shown as evidences may vary.

Certification Authority

- CP/CPS

- Every CA must have a CP/CPS

| Evidence | Method |
| --- | --- |
| CP/CPS | Trivial |

- Every CA must assign its CP/CPS an O.I.D.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 1.2 | Is OID assigned to the CP/CPS |
| Sections in 3647 | 1.2 |  |
| End entity certificate |  | Does EE cert. have PolicyID v3 extension which is set to an OID? |
| OID registry (e.g. IANA) |  | Is OID correct? |

- Whenever there is a change in the CP/CPS the O.I.D. of the document must change and the major changes must be announced to the responsible PMA and approved before signing any certificates under the new CP/CPS.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 8.1 | Does the CP/CPS describe the CP/CPS change procedures, publication and notification policies, and approval procedures? |
| Sections in 3647 | 9.12 |  |
| Interview |  | Ask for details of the CP/CPS administration. For example, who makes changes and who makes decision (approval)? |

- All the CP/CPS under which valid certificates are issued must be available on the web.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.6.1 | Does the CP/CPS describe that all the CP/CPSes under which valid certificates are issued are available on the web? |
| Sections in 3647 | 2.2, 4.4.2, 4.4.3, 4.6.6, 4.6.7, 4.7.6, 4.7.7, 4.8.6, 4.8.7 |  |
| Web repository |  | Are all the CP/CPSes available on the web? |

- The CP/CPS documents should be structured as defined in RFC 3647.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 1.1 | Does the CP/CPS describe that the CP/CPS is structured as defined in RFC 3647? |
| Sections in 3647 | 1.1 |  |
| CP/CPS |  | Is the CP/CPS structured as defined in RFC 3647? |

- CA System

- The CA system must be a dedicated machine.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 6.5.1 | Is the CA system a dedicated machine? |
| Sections in 3647 | 6.5.1 |  |
| Inspection |  | CA system |

- The CA system must be located in a secure environment where access is controlled.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 5.1.1, 5.1.2 | Is the CA system located in a secure environment where access is controlled? |
| Sections in 3647 | 5.1.1, 5.1.2 |  |
| Interview |  | Ask for details of access control to the CA system and its location. For example, who can access to the CA system? How is the access controlled? Is a single person allowed to access to the CA system? How is the access log recorded? |
| Inspection |  | Location of the CA system |

- The CA system must be completely off line or use at least a FIPS 140-2 level 3 Hardware Security Module or equivalent to protect the private key of CA.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 6.1.8, 6.2.1, 6.7 | Is the CA system completely off-line or does it use an HSM? |
| Sections in 3647 | 6.1.1, 6.2.1, 6.7 |  |
| HSM manual |  | Is the HSM at least FIPS 140-2 level 3? |
| Inspection |  | CA system |

- The secure environment must be documented and approved by the PMA, and that document or an approved audit thereof must be available to the PMA.

Can be covered by the auditing item (7).

- CA Key

- The CA key must have a minimum length of 2048 bits

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 6.1.5 | Is the CA key length 2048 bit? |
| Sections in 3647 | 6.1.5 |  |
| Certificate Profile |  | Is the CA key length 2048 bit? |
| CA Certificate |  | Is the CA key length 2048 bit? |

- If the private key of the CA is software-based, it must be protected with a pass phrase of at least 15 elements and it must be known only to designated personnel of the CA.  On-line CAs using an HSM must adopt a similar or better level of security.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 6.2.7 | Does the CPS describe the protection of the CA private key? |
| Sections in 3647 | 6.2.8 |  |
| Interview |  | Ask CA operators who knows the pass phrase. Recommend that CA operators implement multi-person control. |

- Copies of the encrypted private key must be kept on offline media in a secure location where access is controlled.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 6.2.4 | Is the CA private key backup in offline medium? |
| Sections in 3647 | 6.2.4 |  |
| Inspection |  | Backup media and location. |

- The pass phrase of the encrypted private key must also be kept on offline media, separated from the encrypted private keys and guarded in a secure location where only the authorized personnel of the CA have access.  Alternatively, another documented procedure that is equally secure may be used.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 6.2.4, 6.2.5 | Is the CA private key backup in offline medium? |
| Sections in 3647 | 6.2.4, 6.2.5 |  |
| Inspection |  | Backup media and location. |

- The on-line CA architecture should provide for a (preferably tamper-protected) log of issued certificates and signed revocation lists.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.6.1, 4.6.3 | Does the on-line CA provide a log of issued certificates and a signed revocation list? Is the log tamper-protected? |
| Sections in 3647 | 5.5.1, 5.5.3 |  |
| Inspection |  | Log of issued certificates and signed revocation list. |

- When the CA’s cryptographic data needs to be changed, such a transition shall be managed; from the time of distribution of the new cryptographic data, only the new key will be used for certificate signing purposes.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.2, 4.7 | How does the CPS describe transition of the CA’s cryptographic data? |
| Sections in 3647 | 3.3.1, 4.6, 4.7, 5.6 |  |
| End entity certificates (if there was a transition of the CA’s cryptographic data) |  | Is the new EE cert. signed by the new cryptographic data? |

- The overlap of the old and new key must be at least the longest time an end-entity certificate can be valid. The older but still valid certificate must be available to verify old signatures – and the secret key to sign CRLs – until all the certificates signed using the associated private key have also expired.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.2, 4.4.7 | How does the CPS describe transition of the CA’s cryptographic data? |
| Sections in 3647 | 3.3.1, 4.6, 4.7, 5.6 |  |
| End entity certificates Older CA certificate and private key (if there was a transition of the CA’s cryptographic data) |  | Are new EE certificates signed by a new cryptographic data? Is the old but still valid certificate available if there are still valid certificates signed by the old private key? |

- CA Certificate

- Lifetime of the CA certificate must be no longer than 20 years.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.7 | How long is the lifetime of the CA certificate? |
| Sections in 3647 | 5.6 |  |
| CA certificate |  | Check the lifetime of the CA certificate. |

- Lifetime of the CA certificate must be no less than two times of the maximum life time of an end entity certificate.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.7 | How long are the lifetimes of the CA certificate and end entity certificate? |
| Sections in 3647 | 5.6 |  |
| CA certificate and end entity certificate |  | Check the lifetime of the CA certificate and end entity certificate. |

- The CA certificate must have the extension basicConstraints marked as critical.

    - Value of x509 Basic Constraints must be

      - CA:TRUE

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 7.1.2 | Is x509 Basic Constraints set and marked as critical? Is x509 Basic Constraints set to CA:TRUE? |
| Sections in 3647 | 7.1.2 |  |
| CA certificate |  | Check x509 Basic Constraints of the CA certificate. |

- The CA certificate must have the extension keyUsage and it should be marked as critical.

    - Value of x509 Key Usage must include at least

      - keyCertSign, cRLSign

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 7.1.2 | Is x509 Key Usage set and marked as critical? If not, the reason must be explained. Is x509 Key Usage includes keyCertSign and cRLSign?? |
| Sections in 3647 | 7.1.2 |  |
| CA certificate |  | Check x509 Key Usage of the CA certificate. |

- The profile of the CA certificates must also comply with the current IGTF and OGF certificate profile guidelines before being included in any distribution of certificates.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 7.1 | Check the profile of the CA certificate (details are described in the OGF Grid Certificate Profile Document). |
| Sections in 3647 | 7.1 |  |
| CA certificate |  | Check profile of the CA certificate (details should be descried in the OGF Grid Certificate Profile document). |

- Certificate Revocation

- Certificate revocation can be requested by users, the registration authorities, and the CA.  Others can request revocation if they can sufficiently prove compromise or exposure of the associated private key.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.4.2 | Who can request revocation? |
| Sections in 3647 | 4.8.2, 4.9.2 |  |

- The CA must react as soon as possible, but within one working day, to any revocation request received.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.4.3 | How the CA react to revocation requests? |
| Sections in 3647 | 4.9.5 |  |

- An end entity must request revocation of its certificate as soon as possible, but within one working day after detection of

    - he/she lost or compromised the private key pertaining to the certificate,

    - the data in the certificate are no longer valid.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.1.3, 4.4.1 | Does an end entity obligation include requesting revocation if she/he lost or compromised the private key or any data in the certificate is no longer valid? |
| Sections in 3647 | 4.9.1 |  |

- Revocation requests must be properly authenticated.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.4.3 | How is a revocation request authenticated? |
| Sections in 3647 | 4.9.3 |  |
| Interview |  | Ask for details of the revocation process. |

- Certificate Revocation List (CRL)

- Every CA must generate CRLs.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.1.1 | Does the CA issues CRLs? |
| Sections in 3647 | 4.9.7 |  |
| Web repository |  | Are CRLs available on the web? |

- The CRL lifetime must be no more than 30 days.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.4.9 | How long is the lifetime of the CRL? |
| Sections in 3647 | 4.9.9 |  |
| Issued CRLs |  | Is the lifetime of a CRL less than 30 days? |

- Every CA must issue a new CRL at least 7 days before expiration.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.4.9 | Is a new CRL issued at least 7 days before expiration? |
| Sections in 3647 | 4.9.9 |  |
| Issued CRLs |  | Is a CRL issued at least 7 days before expiration? |

- Every CA must issue a new CRL immediately after a revocation.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.4.9 | Is a new CRL issued immediately after a revocation? |
| Sections in 3647 | 4.9.9 |  |
| Interview |  | How does the CA issue a CRL if it receives multiple revocation requests simultaneously? |
| Issued CRLs |  | Check an issued CRL to confirm that a CRL issued immediately after a revocation. |

- The signed CRL must be published in a repository at least accessible via a Web.

Can be covered by the auditing item (25).

- The profile of the CRL must also comply with the current IGTF and OGF certificate profile guidelines before being included in any distribution of certificates.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 7.2 | Is the profile of the CRL compliant with the current IGTF and OGF certificate profile? |
| Sections in 3647 | 7.2 |  |
| Issued CRL |  | Is the profile of the CRL compliant with the current IGTF and OGF certificate profile? |

- The CRLs must be compliant with RFC3280, and is recommended to be version 2.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 7.2.1 | Is the CRL compliant with RFC 3280? What is the version of the CRL? |
| Sections in 3647 | 7.2.1 |  |
| Issued CRL |  | Is the CRL compliant with RFC 3280? What is the version of the CRL? |

- The message digests of the certificates and CRLs must be generated by a trustworthy mechanism, like SHA1 (in particular, MD5 must not be used).

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 7.2 | How the message digests of the certificate and CRLs generated? |
| Sections in 3647 | 7.2 |  |
| End entity certificates and issued CRL |  | How the message digests of the certificate and CRLs generated? |

- End Entity Certificates and keys

- The user key and the host key must have a minimum length of 1024 bits.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 6.1.5 | Is the length of user/host keys at least 1024 bit? |
| Sections in 3647 | 6.1.5 |  |
| Certificate Profile |  | Is the length of user/host keys at least 1024 bit? |
| User and host Certificate |  | Is the length of user/host keys at least 1024 bit? |

- Lifetime of user certificates and host certificates must be no longer than 13 months.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.7 | How long is the lifetime of user and a host certificates? |
| Sections in 3647 | 5.6 |  |
| CA certificate |  | Check the lifetime of user and host certificates. |

- No user certificates may be shared.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.1.3 | Is this described as an end-entity obligation? |
| Sections in 3647 | 4.5.1 |  |

- Each host certificate must be linked to a single network entity.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.1.2 | Does the CP/CPS describe how each host certificate is linked to a single network entity? |
| Sections in 3647 | 3.1.2, 3.1.3 |  |
| Host certificate |  | Does the subject name represent a single network entity? |

- The authority shall issue X.509 certificates to end entities based on cryptographic data generated by the applicant, or based on cryptographic data that is be held only by the applicant on a secure hardware token.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.1, 6.1.1 | How is an end entity’s key generated? |
| Sections in 3647 | 4.1, 4.2 |  |
| Users manual |  | How is an end entity’s key generated? |
| Interview |  | Ask CA operators to demonstrate the generation of a CSR. |

- Every CA should make a reasonable effort to make sure that end-entities realize the importance of properly protecting their private data.  When using software tokens, the user must protect user’s private key with a strong pass phrase, i.e., at least 12 characters long and following current best practice in choosing high-quality passwords.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 6.2.7 | Is this described as an end-entity obligation? |
| Sections in 3647 | 6.2.8 |  |

- The end-entity certificates must be in X.509v3 format and compliant with RFC3280 unless explicitly stated otherwise. In the certificate extensions:

    - a Policy Identifier must be included and must contain an OID and an OID only

    - CRLDistributionPoints must be included and contain at least one http URL

    - keyUsage must be included and marked as critical

    - basicConstraints should be included and it must be set to ‘CA: false’ and marked as critical

    - if an OCSP responder, operated as a production service by the issuing CA, is available, AuthorityInfoAccess must be included and contain at least one URI

    - for certificates bound to network entities, a FQDN shall be included as a dnsName in the SubjectAlternativeName

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 7.1 | Do the X.509 v3 extensions conform to these requirements? |
| Sections in 3647 | 7.1 |  |
| Certificate Profile (if there is a separate document) |  | Do the X.509 v3 extensions conform these requirements? |
| End entity certificates |  | Do the X.509 v3 extensions conform these requirements? |

- The profile of the end entity certificates must also comply with the current IGTF and OGF certificate profile guidelines before being included in any distribution of certificates.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 7.1 | Check the profile of the EE certificates (details are described in the OGF Grid Certificate Profile Document). |
| Sections in 3647 | 7.1 |  |
| Certificate Profile (if there is a separate document) |  | Check the profile of the EE certificates (details are described in the OGF Grid Certificate Profile Document). |
| End entity certificates |  | Check the profile of the EE certificates (details are described in the OGF Grid Certificate Profile Document). |

- If a commonName component is used as part of the subject DN, it should contain an appropriate presentation of the actual name of the end-entity.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.1.2 | Does the CPS describe need for names to be meaningful? |
| Sections in 3647 | 3.1.2, 3.1.3 |  |
| End entity certificates |  | Check end entity certificates. |

- Certificates (and private keys) managed in a software token should only be re-keyed, not renewed.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.2, 4.7 | How are the re-key and re-new processes described? |
| Sections in 3647 |  |  |
| Users manual |  | How are the re-key and re-new processes described? |

- Certificates associated with a private key residing solely on hardware token may be renewed for a period of up to 5 years (for equivalent RSA key lengths of 2048 bits) or 3 years (for equivalent RSA key lengths of 1024 bits).

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.2, 4.7 | How is the re-new process described? |
| Sections in 3647 | 3.3.1, 4.6, 4.7, 5.6 |  |
| Users manual |  | How is the re-new process described? |

- Certificates may be renewed or re-keyed for more than 5 years without a form of identity and eligibility verification, and this procedure must be described in the CP/CPS.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.2, 4.7 | How are the re-key and re-new processes described? |
| Sections in 3647 | 3.3.1, 4.6, 4.7, 5.6 |  |
| Users manual |  | How are the re-key and re-new processes described? |

- Records Archival

- Every CA must record and archive all requests for certificates, along with all issued certificates, all requests for revocation, all the issued CRLs and login/logout/reboot information of the issuing machine.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.6.1 | Does the CA record and archive all requests for certificates, along with all issued certificates, all request for revocation, all issued CRLs and login/logout/reboot information of the issuing machine? |
| Sections in 3647 | 5.5.1 |  |
| Inspection |  | Archived logs |

- These records must be available to external auditors in the course of their work as auditor.

Can be covered by auditing item (46).

- These records must be kept for at least three years, where the identity validation records must be kept at least as long as there are valid certificates based on such a validation.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.6.2 | Is the archive kept at least three years? Is the identity validation record kept at least as long as there are valid certificates based on such a validation? |
| Sections in 3647 | 5.5.2 |  |
| Inspection |  | Archived logs |

- Audits

- Each CA must accept being audited by other accredited CAs to verify its compliance with the rules and procedures specified in its CP/CPS document.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.7 | Does the CA accept external auditing? How is the procedure of auditing described in the CP/CPS? |
| Sections in 3647 | 8 |  |

- Every CA must perform operational audits of the CA/RA staff at least once per year.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.5 | How does the CA perform operational audits? |
| Sections in 3647 | 5.4 |  |
| Operational manual |  | How does the CA perform operational audits? |
| Interview |  | Ask CA operators the details of operational audit. |

- A list of CA and RA personnel should be maintained and verified at least once per year.

| Evidence | Method |
| --- | --- |
| A list of CA and RA personnel | Is the list appropriately maintained? |

- Publication and Repository responsibilities

- The repository must be run at least on a best-effort basis, with an intended availability of 24x7.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.6.4 | Is the web repository available 24x7 on a best effort basis? |
| Sections in 3647 | 2.1 |  |
| Web repository |  | Is the web repository available? |

- The accredited authority must publish their X.509 signing certificate as the root of trust.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.6.1 | Is the CA root certificate published? |
| Sections in 3647 | 2.2, 4.4.2 |  |
| Web repository |  | Is the CA root certificate published? |

- Each authority must publish the following for their subscribers, relying parties and for the benefit of distribution by the PMA and the federation

    - the CA root certificate or set of CA root certificates up to a self-signed root;

    - a http or https URL of the PEM-formatted CA certificate;

    - a http URL of the PEM or DER formatted CRL;

    - a http or https URL of the web page of the CA for general information;

    - the CP and/or CPS documents;

    - an official contact email address for inquiries and fault reporting

    - a physical or postal contact address

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.6.1 | Is this information published? |
| Sections in 3647 | 2.2, 4.4.2, 4.6.6, 4.7.6, 4.8.6 |  |
| Web repository |  | Is this information published? |

- The originating authority must grant to the PMA and the Federation – by virtue of its accreditation – the right of unlimited re-distribution of this information.

| Evidence | Method |
| --- | --- |
| Re-distribution sites | Is this information re-distributed? |

- The CA should provide a means to validate the integrity of its root of trust.

| Evidence | Method |
| --- | --- |
| Web repository | Does the CA provide a means to validate the integrity of its root of trust? |

- The CA shall provide their trust anchor to a trust anchor repository, specified by the accrediting PMA, via the method specified in the policy of the trust anchor repository.

| Evidence | Method |
| --- | --- |
| Trust anchor repository | Does the CA provide their trust anchor? |

- Privacy and confidentiality

- Accredited CAs must define a privacy and data release policy compliant with the relevant national legislation. The CA is responsible for recording, at the time of validation, sufficient information regarding the subscribers to identify the subscriber. The CA is not required to release such information unless provided by a valid legal request according to national laws applicable to that CA.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.8 | How are privacy and confidentiality described? |
| Sections in 3647 | 9.3, 9.4 |  |

- Compromise and disaster recovery

- The CA must have an adequate compromise and disaster recovery procedure, and we willing to discuss this procedure in the PMA. The procedure need not be disclosed in the policy and practice statements.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.8 | How are procedures of compromise and disaster recovery described? |
| Sections in 3647 | 5.7, 5.7.1 |  |
| Interview |  | Ask CA operators the detailed procedures of compromise and disaster recovery. |

Registration Authority

- Entity Identification

- A PKI CA must define the role of a registration authority (RA), and these RAs are responsible for the identity vetting of all end entities.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.1.2, 4.1 | What is the role of the RA? |
| Sections in 3647 | 4.1, 4.2, 4.6, 4.7 |  |

- In order for an RA to validate the identity of a person, the subject should contact the RA face-to-face and present photo-id and/or valid official documents showing that the subject is an acceptable end entity as defined in the CP/CPS document of the CA.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.1.2, 4.1 | How does an RA implement identity vetting? |
| Sections in 3647 | 4.1, 4.2, 4.6, 4.7 |  |
| Operational manual |  | How does an RA identify a person? |
| Interview |  | Ask RA operators the detailed procedure of identity vetting. |

- In case of host or service certificate requests, an RA should validate the identity of the person in charge of the specific entities using a secure method.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.1.2, 4.1 | How does an RA validate the identity of a person requesting a host/service certificate? |
| Sections in 3647 | 4.1, 4.2, 4.6, 4.7 |  |
| Operational manual |  | How does an RA identify a person requesting a host/service certificate? |
| Interview |  | Ask RA operators the detailed procedure of identity vetting for host/service certificate requests. |

- An RA should ensure that the requestor is appropriately authorized by the owner of the FQDN or the responsible administrator of the machine to use the FQDN identifiers asserted in the certificate.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 2.1.2, 4.1 | How does an RA ensure that the requestor is appropriately authorized by the owner of the FQDN? |
| Sections in 3647 | 4.1, 4.2, 4.6, 4.7 |  |
| Operational manual |  | How does an RA ensure that the requestor is appropriately authorized by the owner of the FQDN? |
| Interview |  | Ask RA operators the detailed procedure of identity vetting. |

- Name Uniqueness

- Any single subject distinguished name must be linked to one and only one entity.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.1.4 | How does the CA guarantee the uniqueness of the subject name? |
| Sections in 3647 | 3.1.5 |  |
| Interview |  | How does the CA guarantee the uniqueness of the subject name? What happens if there are two persons whose names are the same in the same organization? |

- Over the entire lifetime of the CA it must not be linked to any other entity.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 3.1.4 | How does the CA guarantee this requirement? |
| Sections in 3647 | 3.1.5 |  |
| Interview |  | Ask for the details of the method to guarantee this requirement. |

- RA to CA communications

- The RA must communicate with the CA using secure methods that are clearly defined in the CP/CPS.  (e.g. signed emails, voice conversations with a known person, or some other acceptable method)

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.1, 4.2 | How does the CA communicate with the RA? |
| Sections in 3647 | 4.1, 4.2 |  |
| Operational manual |  | How does CA communicate with the RA? |
| Interview |  | Ask tfor he details of how the CA communicates with the RA (e.g. how the CSR is sent to the CA and the signed certificate is sent to the RA). |

- The CP/CPS should describe how the RA or CA is informed of changes that may affect the status of the certificate.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.4 | How is the CA or the RA informed of changes? |
| Sections in 3647 | 4.8, 4.9 |  |
| Interview |  | Ask for the details of how the CA or the RA is informed of change. |

- Records and Archival

- The RA must record and archive all requests and confirmations.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.6.1 | Does the RA record and archive all requests and confirmations? |
| Sections in 3647 | 5.5.1 |  |
| Inspection |  | Archives of all requests and confirmations. |

- The CA is responsible for maintaining an archive of these records in an auditable form.

| Evidence |  | Method |
| --- | --- | --- |
| Sections in 2527 | 4.6 | Does the RA maintain the archive of these records in an auditable form? |
| Sections in 3647 | 5.5.1 |  |
| Inspection |  | Archives of these records archival. |

- Security Considerations

The IGTF defines Authentication Profiles and each member PMA is responsible for accrediting member Certificate Authorities according to a specific profile.  Once a member CA is accredited by its belonging PMA, it is expected to do self-auditing and/or to accept external auditing to confirm the compliance of the CA.  This document describes guidelines for auditing Grid CAs which comply with the Classic Authentication Profile and the audit checklist is built based on the Classic Authentication Profile Version 4.1.  Since evidences for verifying each audit item may differ between CAs, auditors must carefully verify the audit item with appropriate evidences.  Auditors should especially be interested in the certificate life cycle management, protection of CA’s private key, and logs are archived enough to trace anything when something would happen.

Contributors

This document captures the collective knowledge of many people, and the editor is grateful for the contributions made to this document by the members of the International Grid Trust Federation [IGTF], the individual certification authorities and relying parties that have conducted the experiments and tests, and the valuable contributions from the participants in the CAOPS WG.

The editors,
Yoshio Tanaka (yoshio.tanaka@aist.go.jp)

Matthew Viljoen (M.J.Viljoen@rl.ac.uk)

Intellectual Property Statement

The OGF takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.  Copies of claims of rights made available for publication and any assurances of licenses to be made available, or the result of an attempt made to obtain a general license or permission for the use of such proprietary rights by implementers or users of this specification can be obtained from the OGF Secretariat.

The OGF invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights which may cover technology that may be required to practice this recommendation.  Please address the information to the OGF Executive Director.

Disclaimer

This document and the information contained herein is provided on an “As Is” basis and the OGF disclaims all warranties, express or implied, including but not limited to any warranty that the use of the information herein will not infringe any rights or any implied warranties of merchantability or fitness for a particular purpose.

Full Copyright Notice

Copyright (C) Open Grid Forum (2007). All Rights Reserved.

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works. However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the OGF or other organizations, except as needed for the purpose of developing Grid Recommendations in which case the procedures for copyrights defined in the OGF Document process must be followed, or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by the OGF or its successors or assignees.

References

[1] S. Chokhani and W. Ford, “Internet X.509 Public Key Infrastructure Certificate Policy and Certification Practices Framework”, RFC2527, The Internet Society, 1999.

[2] S. Chokhani, W. Ford, R. Sabett, C. Merrill, and S. Wu, “Internet X.509 Public Key Infrastructure Certificate Policy and Certification Practices Framework”, RFC3647, The Internet Society, 2003.

[3] D. Groep, “Authentication Profile for Classic X.509 Public Key Certification Authorities with secured infrastructure”, International Grid Trust Federation, IGTF-AP-classic-4-1, 2006.

As of October 17, 2007, Grid Certificate Profile document does not describe CRL profiles.